Hello everyone,

Today, we are going to look at how to connect and set up the Expert Automatics Water Level Controller with a Single Phase 1HP MCP Type Starter.

So, if you look at the Expert Automatics Water Level Controller, you will find input, output, and some common tank terminals.

In this, you have P (Phase) and N (Neutral) terminals in the input section.

- P is for Phase

- N is for Neutral

Similarly, in the output section, we will connect the necessary wires.

Connecting the Starter

For this setup, I take two wires and connect them as required.
As I mentioned earlier, first, I will connect the neutral wire in the output section.

Next, I will connect the input phase wire.

That's all for the connections.

After this, we will connect the RVB wire to the motor.
Then, turn on the MCP starter.

Now, let’s see how to connect the float sensor to the controller.

Float Sensor Wiring

The float sensor comes with three wires:

- Red

- Black

- Blue

For the tank, we only need to use the red and black wires:

- Red = Common

- Black = Output

To connect it to the controller, you will find three terminals labeled float, common, and tank.

- The red wire connects to the common terminal.

- The black wire connects to the T (Tank) terminal.

Dry Run Sensor Connection

The Dry Run Sensor requires two wires.

- One wire connects to the common terminal.

- The other wire connects to the S (Sensor) terminal.

Now, let’s check the probe connections.

Testing the Setup

After powering on the controller, you will see four LED indicators:

- Power ON LED

- Motor ON LED

- Tank FULL LED

- Dry Run LED

I will now switch to manual mode.

Once in manual mode, you will see the Motor ON LED light up, indicating that the motor is running.

Now, let’s switch to Auto Mode.

- If the water level is low, the buzzer will sound for 5 seconds, and then the motor will turn ON.

- Once the tank is full, the Tank FULL LED will turn ON, and the motor will automatically turn OFF.

If the tank gets empty again, the buzzer will sound for 5 seconds, and the motor will turn ON.
When the tank is full, the Tank FULL LED will glow, and the motor will turn OFF.

Dry Run Protection

Now, let’s test the dry run protection.

- If the tank is empty and the motor runs without water, the buzzer will sound for 5 seconds, and the motor will turn OFF.

- After 50 seconds, the Dry Run LED will turn ON, indicating the motor is turned OFF to prevent damage.

To restart the system, you must turn the motor OFF manually and then switch it back to Auto Mode.

If you notice that the motor is running but there is no water, you should disconnect the wiring, wait for 50 seconds, and check if the Dry Run LED turns ON. If so, the motor will automatically turn OFF to prevent damage.

This is how you set up and test the Expert Automatics Water Level Controller with a Single Phase 1HP MCP Type Starter.